QUESTION 1

Construct a pipeline scenario containing dependent instructions, branch instructions, and competing resource requirements, and apply techniques such as pipeline stalling, operand forwarding, branch prediction, and resource duplication to minimize performance losses.

ANSWER –

Design of a Pipeline Scenario

Consider a processor with a 5-stage instruction pipeline consisting of the following stages:

- IF (Instruction Fetch): Fetches the instruction from memory.

- ID (Instruction Decode): Decodes the instruction and reads the required registers.

- EX (Execute): Performs arithmetic or logical operations and calculates memory addresses.

- MEM (Memory Access): Reads from or writes to memory for load/store instructions.

- WB (Write Back): Stores the result back into the destination register.

The processor executes the following instruction sequence

I1: ADD R1, R2, R3

I2: SUB R4, R1, R5

I3: LW  R6, 0(R4)

I4: BEQ R6, R0, LABEL

I5: OR  R7, R6, R8

This instruction sequence contains data dependency, branch instructions, and resource conflicts, making it suitable for analysing pipeline performance.

Pipeline Hazard Analysis

1. Data Hazard

A data hazard occurs when an instruction depends on the result of a previous instruction that has not yet completed.

- I2 requires the value of R1, which is produced by I1.

- I5 requires the value of R6, which is loaded from memory by I3.

If these dependent instructions execute immediately, incorrect data may be used, resulting in incorrect program execution.

2. Control Hazard

Instruction I4 is a branch instruction.

The processor does not know whether the branch condition is true until the instruction reaches the Execute stage. During this time, the processor may fetch incorrect instructions, reducing pipeline efficiency.

3. Structural Hazard

Assume the processor has a single memory unit shared by both instruction fetch and data access.

When I3 performs a memory read operation, the instruction fetch stage also requires access to memory. Since only one memory is available, both operations cannot occur simultaneously, creating a structural hazard.

Techniques to Minimize Performance Loss

1. Pipeline Stalling

Pipeline stalling introduces temporary wait cycles whenever the required data is unavailable. For example, I2 waits until I1 produces the value of R1 before execution continues.

Although stalling guarantees correct execution, it increases the number of clock cycles and decreases overall processor performance.

2. Operand Forwarding

Operand forwarding transfers the ALU result directly from the Execute stage to the next dependent instruction without waiting for the Write Back stage.

For example, the result generated by I1 is forwarded directly to I2, allowing the instruction to execute without unnecessary waiting.

This technique significantly reduces data hazard stalls and improves instruction throughput.

3. Branch Prediction

To reduce delays caused by branch instructions, the processor predicts whether the branch will be taken or not taken before the actual condition is evaluated.

- If the prediction is correct, instruction execution continues without interruption.

- If the prediction is incorrect, the incorrectly fetched instructions are discarded, and the correct instructions are fetched.

Effective branch prediction minimizes control hazards and improves pipeline efficiency

4. Resource Duplication

Structural hazards are eliminated by providing separate hardware resources for instruction and data memory.

Using separate instruction memory and data memory allows:

- Instruction fetch and memory access to occur simultaneously.

- No competition for memory resources.

- Better utilization of the pipeline stages.

This approach increases processor throughput and reduces execution delays.

Performance Analysis

By applying the above techniques:

- Pipeline stalling ensures correct execution during unavoidable dependencies.

- Operand forwarding removes most data hazard delays.

- Branch prediction reduces the penalty associated with branch instructions.

- Resource duplication eliminates structural hazards caused by shared hardware resources.

As a result, the processor executes more instructions in less time, reduces idle pipeline stages, improves instruction throughput, and achieves higher overall performance compared to a basic pipelined processor without these optimization techniques.

QUESTION 2 -

Develop a computer system model using a hierarchical bus structure to transfer data among the CPU, memory, and I/O devices, and analyse how bus priority, bandwidth, and transfer conflicts affect overall system performance.

ANSWER –

Design of a Hierarchical Bus Structure

A computer system can be designed using a hierarchical bus structure, where different buses are used to connect the CPU, memory, and I/O devices. Instead of using a single shared bus, multiple buses are organized in levels to improve data transfer efficiency and reduce communication delays.

The proposed system model consists of:

- CPU – Executes instructions and controls all data transfers.

- Cache Bus – Provides high-speed communication between the CPU and cache memory.

- Memory Bus – Transfers data between the CPU and the main memory (RAM).

- I/O Bus – Connects peripheral devices such as the keyboard, printer, storage devices, and network interfaces.

Data Transfer Process

- The CPU fetches instructions and data through the Cache Bus.

- If the required data is not available in the cache, the request is sent to the Main Memory through the Memory Bus.

- Input and output operations are performed through the I/O Bus, which transfers data between peripheral devices and the CPU.

- Each bus operates independently, reducing unnecessary delays and allowing efficient communication between system components.

Performance Analysis

1. Bus Priority

When multiple devices request the bus at the same time, a priority mechanism determines which device gains access first.

For example:

- CPU – Highest Priority

- Main Memory – Medium Priority

- I/O Devices – Lower Priority

Higher-priority devices receive bus access immediately, ensuring faster execution of critical operations. However, lower-priority devices may experience increased waiting time if the bus remains occupied by high-priority requests.

2. Bus Bandwidth

Bus bandwidth represents the amount of data that can be transferred per second through the bus.

A wider bus and higher clock frequency increase bandwidth, allowing larger amounts of data to be transferred in less time.

Higher bandwidth results in:

- Faster memory access

- Improved CPU performance

- Reduced data transfer delays

- Better overall system efficiency

3. Transfer Conflicts

Transfer conflicts occur when multiple devices attempt to use the same bus simultaneously.

For example, while the CPU is accessing the main memory, a disk controller may also request the Memory Bus for data transfer. Since only one device can use the bus at a time, one request must wait until the other transfer is completed.

These conflicts increase bus waiting time and reduce system throughput.

Bus arbitration techniques and the use of separate buses help minimize such conflicts.

Question 3

Develop a processor model that supports multiple instruction issue, out-of-order execution, and speculative execution, and analyse how instruction scheduling, execution units, and simultaneous multithreading affect instruction throughput and processor resource utilization.

ANSWER –

Design of the Processor Model

A high-performance processor can be designed to support multiple instruction issue, out-of-order execution, speculative execution, and simultaneous multithreading (SMT). These features allow the processor to execute several instructions efficiently while minimizing idle hardware resources.

The processor model consists of the following components:

- Instruction Fetch Unit – Fetches multiple instructions in each clock cycle.

- Instruction Decode Unit – Decodes instructions and identifies dependencies.

- Instruction Scheduler – Selects instructions that are ready for execution.

- Execution Units – Includes Arithmetic Logic Unit (ALU), Floating Point Unit (FPU), Load/Store Unit, and Branch Unit.

- Reorder Buffer (ROB) – Ensures instructions are committed in the correct program order.

- Register File – Stores operands and execution results.

Processor Features

1. Multiple Instruction Issue

The processor fetches and issues more than one instruction during a single clock cycle. Independent instructions are sent to different execution units simultaneously, increasing instruction throughput and making better use of processor resources.

2. Out-of-Order Execution

Instead of executing instructions strictly in program order, the processor executes any instruction whose operands are ready. Instructions waiting for data are temporarily skipped, while other independent instructions continue execution. The final results are committed in the correct order using the Reorder Buffer.

This approach reduces pipeline stalls and improves execution efficiency.

3. Speculative Execution

For branch instructions, the processor predicts the most likely execution path and begins executing those instructions before the branch decision is confirmed.

- If the prediction is correct, execution continues without interruption.

- If the prediction is incorrect, the speculative results are discarded, and execution resumes from the correct path.

This technique minimizes delays caused by branch instructions.

Performance Analysis

1. Instruction Scheduling

The instruction scheduler continuously checks instruction dependencies and dispatches only those instructions whose operands are available.

Efficient scheduling:

- Reduces waiting time.

- Minimizes pipeline stalls.

- Increases processor throughput.

2. Execution Units

The processor contains multiple execution units such as the ALU, FPU, Load/Store Unit, and Branch Unit. Since different instruction types can execute simultaneously, hardware utilization is improved, and more instructions are completed per clock cycle.

3. Simultaneous Multithreading (SMT)

SMT allows multiple threads to share the same processor core. When one thread is waiting for memory or data, another thread can utilize the available execution units.

This:

- Improves processor utilization.

- Reduces idle cycles.

- Increases overall instruction throughput.

Question 4-

Design and simulate a processor data path that performs register-to-register data transfers and arithmetic operations such as increment, decrement, addition, and subtraction, and analyse the control signals and execution sequence involved in each operation.

ANSWER -

Design of the Processor Datapath

A processor data path is designed to transfer data between registers and perform arithmetic operations using an Arithmetic Logic Unit (ALU). The data path consists of a Register File, Multiplexers (MUXs), ALU, and Control Unit. The Control Unit generates the necessary control signals to select registers, choose the ALU operation, and store the result in the destination register.

Arithmetic Operations

1. Register-to-Register Data Transfer

The contents of one register are copied directly to another register.

Example:

R2 ← R1

The control unit selects R1 as the source register and R2 as the destination register. The data is transferred without any arithmetic operation.

2. Increment Operation

The ALU increases the value stored in a register by one.

Example:

R1 ← R1 + 1

The register value is sent to the ALU, which performs the increment operation and stores the updated value back into R1.

3. Decrement Operation

The ALU decreases the value stored in a register by one.

Example:

R1 ← R1 - 1

The processor reads the register value, performs the subtraction of one, and writes the result back into the same register.

4. Addition Operation

The ALU adds the contents of two registers.

Example:

R3 ← R1 + R2

The values of R1 and R2 are read simultaneously, added by the ALU, and the result is stored in R3.

5. Subtraction Operation

The ALU subtracts one register value from another.

Example:

R3 ← R1 - R2

The ALU performs the subtraction operation, and the result is written back to the destination register.

Control Signals

The Control Unit generates the required control signals for each operation.

- Read Enable: Reads data from the selected source registers.

- Write Enable: Stores the ALU result into the destination register.

- MUX Select: Chooses the input source for the ALU.

- ALU Control: Specifies the required operation such as increment, decrement, addition, or subtraction.

- Register Select: Identifies the source and destination registers involved in the operation.

These control signals ensure that each instruction is executed correctly.

Execution Sequence

The execution of each operation follows these steps:

- The instruction is fetched and decoded.

- The required source registers are selected.

- The register values are sent to the ALU.

- The ALU performs the specified arithmetic or data transfer operation.

- The result is written back to the destination register.

This sequence is repeated for every instruction executed by the processor.

Question 5

Simulate a pipelined processor executing a sequence of arithmetic, memory, and branch instructions, and calculate the pipeline execution time, throughput, and speedup by comparing the results with sequential instruction execution.

ANSWER –

Design of the Pipeline Simulation

Consider a 5-stage pipelined processor with the following stages:

- IF (Instruction Fetch)

- ID (Instruction Decode)

- EX (Execute)

- MEM (Memory Access)

- WB (Write Back)

The processor executes the following instruction sequence:

I1: ADD R1, R2, R3

I2: SUB R4, R1, R5

I3: LW  R6, 0(R4)

I4: BEQ R6, R0, LABEL

I5: OR  R7, R6, R8

This sequence includes arithmetic, memory, and branch instructions, representing a typical pipelined execution scenario.

Pipeline Execution

The instructions move through the pipeline as shown below:

| Clock Cycle | I1 | I2 | I3 | I4 | I5 |
| --- | --- | --- | --- | --- | --- |
| 1 | IF |  |  |  |  |
| 2 | ID | IF |  |  |  |
| 3 | EX | ID | IF |  |  |
| 4 | MEM | EX | ID | IF |  |
| 5 | WB | MEM | EX | ID | IF |
| 6 |  | WB | MEM | EX | ID |
| 7 |  |  | WB | MEM | EX |
| 8 |  |  |  | WB | MEM |
| 9 |  |  |  |  | WB |

Sequential Execution

In a non-pipelined processor, each instruction completes all five stages before the next instruction begins.

- Number of instructions = 5

- Stages per instruction = 5

Sequential Execution Time

5×5=25 clock cycles

Pipeline Execution Time

For a 5-stage pipeline:

Pipeline Time=Number of Stages+(Number of Instructions−1)

=5+(5−1)

=9 clock cycles

Throughput Calculation

Throughput is the number of instructions completed per clock cycle.

Throughput is the number of instructions completed per clock cycle.

Throughput= Number of Instructions/ Pipeline Execution Time  = 9/5

=0.56 instructions per clock cycle

Speedup Calculation

Speedup compares the performance of pipelined execution with sequential execution.

Thus, the pipelined processor is approximately 2.78 times faster than the sequential processor for this instruction sequence.