# SIMATS ENGINEERING

Department of Computer Science and Engineering

## ASSESSMENT TOOL 1 – CONSTRAINT-BASED PROBLEM SOLVING

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO5 – Precision (BL4) | Assessment: | Constraint Based Problem Solving |
| Weightage: | 40% | Total Marks: | 100 |
| CO5 Statement: | Construct I/O communication mechanisms by comparing memory-mapped and I/O-mapped techniques, interrupt handling (vectored, hardware, software), DMA controllers, and advanced bus interfaces including PCIe, USB, NVMe, and Thunderbolt. |  |  |
| Student Name: | A.KULLAI REDDY | Reg. No.: | 192412647 |
| Section / Batch: | 2024 | Date: | 26/08/2026 |

Instructions to Students

- Identify the relevant concepts of I/O interfaces, interrupts, DMA, bus arbitration, and high-speed communication protocols.

- Analyze the I/O requirements considering CPU utilization, latency, throughput, bandwidth, and device priorities.

- Apply suitable I/O techniques such as DMA, vectored interrupts, memory-mapped I/O, nested interrupts, and bus arbitration.

- Compute performance measures such as data transfer rate, interrupt latency, CPU utilization, throughput, and transfer time.

- Interpret the results by evaluating CPU efficiency, communication performance, interrupt response, and suitability of the proposed I/O architecture.

QUESTIONS

- Design an I/O subsystem for a real-time robotic arm controller with the following constraints:

  - Position sensors generate data every 500 µs.

  - Control commands must be processed within 5 µs.

  - Use priority-based interrupt handling for critical sensors.

  - Non-critical diagnostic data must be transferred using DMA.

- Construct a communication architecture for an autonomous vehicle with the following constraints:

  - Multiple cameras generate a combined data rate of 2 GB/s.

  - Safety-critical sensors require deterministic response time.

  - Use PCIe for high-bandwidth devices.

  - Ensure that low-priority devices do not affect safety-critical communication.

- Design an I/O mechanism for an industrial monitoring system with the following constraints:

  - 100 sensors continuously generate measurement data.

  - The system must support real-time fault detection.

  - Use interrupt coalescing to reduce processor overhead.

  - Critical fault signals must receive immediate interrupt service.

- Construct a high-speed network interface subsystem with the following constraints:

  - Network traffic can reach 10 Gb/s.

  - The CPU must be protected from excessive packet-processing overhead.

  - Use DMA-based packet transfer between the network interface and memory.

  - Generate interrupts only after processing a configurable batch of packets.

- Design an I/O architecture for a smart healthcare monitoring system with the following constraints:

  - Patient-monitoring devices generate data at different sampling rates.

  - Emergency signals must be handled with highest priority.

  - Use nested interrupt handling for urgent events.

  - Store continuous monitoring data using DMA transfers without interrupting critical processing.

Code of Conduct

I 	(Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment. I understand that any violation of academic integrity rules will result in disciplinary action.

## Signature of the Student:

RUBRICS FOR CALCULATION

| Criteria | Weightage(%) | Level 4 – Exemplary (5 Marks) | Level 3 – Proficient (4 Marks) | Level 2 – Developing (2–3 Marks) | Level 1 – Beginning (0–1 Mark) |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 25% | Clearly defines the problem and identifies all relevant constraints accurately | Identifies most constraints with minor gaps | Partial understanding; misses key constraints | Fails to identify constraints correctly |
| Constraint Analysis | 25% | Analyzes constraints effectively and prioritizes them logically | Provides reasonable analysis with some prioritization | Limited analysis; weak prioritization | No meaningful analysis or prioritization |
| Solution Development | 25% | Develops optimal and feasible solutions satisfying all constraints | Develops workable solutions with minor limitations | Solutions partially satisfy constraints | Solutions do not meet constraints |
| Application of Concepts | 15% | Effectively applies appropriate concepts, methods, or tools | Applies concepts with minor errors | Limited application; requires guidance | Unable to apply relevant concepts |
| Justification | 10% | Provides strong justification for decisions with logical reasoning | Provides justification with some clarity | Weak or unclear justification | No justification provided |

## Marks Evaluation:

| Q. No | Problem Understanding (25) |  | Constraint Analysis (25) |  | Solution Development (25) | Application of Concepts (15) | Justification (10) | Total (out of 100) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 |  |
| 2 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 |  |
| 3 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 |  |
| 4 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 |  |
| 5 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 |  |
|  | / 25 |  | /25 |  | / 25 | / 15 | / 10 | / 100 |

Course Coordinator	Faculty Incharge

Answers: I/O Subsystem and Communication Architecture Designs

Q1. I/O Subsystem for a Real-Time Robotic Arm Controller

Design Approach:

Position sensors (every 500 ms) are handled by a high-priority interrupt so control commands can be processed within the 5 ms deadline. A priority-based interrupt controller ensures critical sensor/control interrupts always preempt non-critical work. Diagnostic data, which is not time-critical, is offloaded to DMA so it never consumes CPU cycles needed for control.

Pseudocode:

BEGIN

ASSIGN Priority: Priority(PositionSensorInterrupt) = HIGH Priority(ControlCommandInterrupt) = HIGHEST

Priority(DiagnosticTransfer)	= LOW (handled by DMA, not CPU)

ON PositionSensor_Interrupt (every 500 us): Save minimal context

Read position data quickly Store into ControlBuffer Signal ControlTask

ON ControlCommand_Interrupt (deadline 5 us): Preempt any lower priority activity immediately

Execute control command directly from ISR (time-critical, minimal code path) Send actuator signal

Return from interrupt

DIAGNOSTIC DATA HANDLING (non-critical):

Configure DMA controller:

Source = Diagnostic sensor buffer Destination = Memory log area

START DMA transfer (runs in background, no CPU involvement) ON DMA_Complete_Interrupt (low priority):

Mark diagnostic block as ready for later analysis

SCHEDULER RULE:

IF ControlCommand_Interrupt pending:

Preempt everything except another ControlCommand in progress ELSE IF PositionSensor_Interrupt pending:

Service after any pending control command ELSE:

Let DMA handle diagnostics in background

END

Q2. Communication Architecture for an Autonomous Vehicle

Design Approach:

High-bandwidth camera data (2 GB/s) is routed over PCIe, using DMA transfers so the CPU is not burdened moving large frames. Safety-critical sensors get a separate, dedicated, high-priority channel with guaranteed bandwidth and interrupt priority, isolated from the camera traffic so bulk data transfers can never delay a safety-critical message (bus/channel partitioning).

Pseudocode:

BEGIN

CHANNEL SETUP:

HighBandwidthChannel = PCIe lanes (dedicated to cameras)

SafetyCriticalChannel = separate dedicated bus/lane (isolated from PCIe camera traffic) LowPriorityChannel	= shared bus for diagnostics/logging

CAMERA DATA PATH (2 GB/s):

Configure PCIe DMA engine:

Source = Camera sensor buffers Destination = Shared memory frame buffer

Continuously stream via DMA (no CPU polling) ON DMA_Frame_Complete:

Notify vision-processing task (low-priority interrupt)

SAFETY-CRITICAL SENSOR PATH (deterministic):

Assign HIGHEST interrupt priority to SafetyCriticalChannel ON SafetyCritical_Interrupt:

Preempt ALL other bus/interrupt activity Process sensor data within guaranteed deadline

Bus arbitration ALWAYS grants this channel immediate access

BUS ARBITRATION RULE:

IF SafetyCriticalChannel has pending request:

Grant bus access immediately (reserved bandwidth slice) ELSE:

Allow HighBandwidthChannel (camera DMA) to use remaining bandwidth Low-priority diagnostic traffic only uses leftover idle bandwidth

MONITORING:

Continuously verify SafetyCriticalChannel latency stays within deadline IF violation detected:

Raise alert and throttle camera/diagnostic traffic further

END

Q3. I/O Mechanism for an Industrial Monitoring System

Design Approach:

With 100 continuously reporting sensors, generating one interrupt per reading would overwhelm the CPU. Interrupt coalescing batches normal sensor readings into a single interrupt after a count or time threshold. Critical fault signals bypass coalescing entirely and trigger an immediate, high-priority interrupt for real-time fault detection.

Pseudocode:

BEGIN

CONFIGURE Interrupt Coalescing: CoalesceCountThreshold = N readings (e.g., 50)

CoalesceTimeThreshold = T milliseconds (e.g., 10 ms)

FOR each Sensor S in SensorArray (100 sensors): ON Normal_Reading(S):

Append reading to CoalesceBuffer

IF CoalesceBuffer.count >= CoalesceCountThreshold

OR TimeSinceLastInterrupt >= CoalesceTimeThreshold: Raise single "Batch Data Ready" interrupt

Reset CoalesceBuffer and timer ON Batch_Data_Ready_Interrupt:

Process all buffered readings together (bulk processing, low overhead) Run fault-detection analysis across the batch

CRITICAL FAULT PATH (bypasses coalescing): ON Critical_Fault_Signal(S):

Raise IMMEDIATE high-priority interrupt (no batching, no delay) Preempt normal batch processing

Execute fault handler:

Identify faulty sensor/condition Trigger alarm / safety action Log event with timestamp

RULE:

Critical faults ALWAYS interrupt immediately

Routine measurements ALWAYS coalesced to reduce CPU load

END

Q4. High-Speed Network Interface Subsystem

Design Approach:

At up to 10 Gb/s, per-packet interrupts would overload the CPU, so the network interface card (NIC) uses DMA to move packets directly into memory. Interrupts are generated only after a configurable batch of packets has been transferred (interrupt moderation/coalescing), keeping CPU overhead low while still processing traffic promptly.

Pseudocode:

BEGIN

CONFIGURE NIC:

DMA_RingBuffer = allocate memory region for incoming packets BatchSize = configurable value (e.g., 64 packets) MaxWaitTime = configurable timeout (e.g., 100 microseconds)

ON Packet_Arrival (up to 10 Gb/s):

NIC writes packet directly into DMA_RingBuffer (no CPU involvement) Increment PacketCount

IF PacketCount >= BatchSize OR TimeSinceLastInterrupt >= MaxWaitTime: Raise single "Batch Ready" interrupt to CPU

Reset PacketCount and timer

ON Batch_Ready_Interrupt:

CPU reads batch of packets from DMA_RingBuffer Process packets (parsing, routing, protocol handling) Free/recycle consumed ring buffer slots

Re-enable DMA writes to freed slots

OVERFLOW HANDLING:

IF DMA_RingBuffer becomes full before CPU processes batch: Raise urgent interrupt immediately (override batching) Drop lowest-priority packets if necessary to prevent stall

TUNING:

Adjust BatchSize and MaxWaitTime dynamically based on observed traffic load, to balance latency vs CPU overhead

END

Q5. I/O Architecture for a Smart Healthcare Monitoring System

Design Approach:

Patient-monitoring devices sample at varying rates, so each is assigned an interrupt priority based on urgency. Emergency signals get the highest priority and can preempt an already-executing interrupt using nested interrupt handling. Continuous routine monitoring data (e.g., vital sign logging) is transferred via DMA in the background so it never interrupts critical processing.

Pseudocode:

BEGIN

ASSIGN Priorities:

Priority(EmergencySignal) = HIGHEST (e.g., cardiac arrest, apnea alarms) Priority(UrgentEvent)	= HIGH	(e.g., abnormal vitals) Priority(RoutineSampling) = LOW	(handled via DMA)

NESTED INTERRUPT HANDLING:

ON UrgentEvent_Interrupt (in progress): IF EmergencySignal_Interrupt occurs:

Preempt current UrgentEvent handler Save its context (nested stack)

Service EmergencySignal_Interrupt immediately Trigger alarm / notify medical staff

Resume UrgentEvent handler after emergency handled

ROUTINE MONITORING DATA (continuous, varying sampling rates): FOR each PatientDevice D:

Configure DMA channel for D: Source = D's sensor output

Destination = Patient record buffer in memory Start continuous DMA transfer at D's sampling rate

ON DMA_Complete_Interrupt (low priority): Store data into patient history log

No interruption to emergency/urgent processing

RULE SUMMARY:

Emergency signals always preempt (nested interrupts)

Urgent events serviced with high priority, but can be preempted by emergencies Routine data logged silently via DMA, zero impact on critical response time

END