# SIMATS ENGINEERING

### Department of Computer Science and Engineering

## ASSESSMENT TOOL 2 – PSEUDOCODE WRITING TASK

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO5 – Precision (BL4) | Assessment: | Pseudocode Writing task |
| Weightage: | 30% | Total Marks: | 100 |
| CO5 Statement: | Construct I/O communication mechanisms by comparing memory-mapped and I/O-mapped techniques, interrupt handling (vectored, hardware, software), DMA controllers, and advanced bus interfaces including PCIe, USB, NVMe, and Thunderbolt. |  |  |
| Student Name: | A.KULLAI REDDY | Reg. No.: | 192412647 |
| Section / Batch: | 2024 | Date: | 26/08/2026 |

Instructions to Students

- Identify the relevant concepts of DMA-based data transfer, vectored interrupts, I/O mapping techniques, bus arbitration, and hybrid I/O communication.

- Analyze the requirements of different I/O devices by considering CPU involvement, interrupt priority, latency, bandwidth, throughput, and transfer speed.

- Apply appropriate I/O techniques such as DMA, interrupt-driven I/O, vectored interrupts, memory-mapped I/O, I/O-mapped I/O, and bus arbitration to develop efficient solutions.

- Develop pseudocode algorithms for DMA data transfer, prioritized interrupt handling, dynamic I/O mapping selection, bus arbitration, and hybrid polling–DMA communication.

- Evaluate the proposed I/O mechanisms by interpreting CPU utilization, data transfer efficiency, interrupt response time, communication performance, resource allocation, and overall system suitability.

### QUESTIONS

- A real-time monitoring system receives data from multiple sensors with different sampling frequencies and response-time requirements. Develop a pseudocode for an interrupt scheduling mechanism that assigns suitable interrupt priorities to the sensors based on their sampling rate, urgency, and data-processing requirements. The algorithm should manage simultaneous interrupt requests and ensure that time-critical sensor data is serviced without causing excessive CPU overhead.

- A high-speed data acquisition system must transfer continuous streams of data between an I/O device and memory while preventing data loss when the incoming data rate exceeds the processing speed of the CPU. Write a pseudocode for a double-buffered DMA mechanism that alternates between two memory buffers during data transfer. The algorithm should detect buffer completion, switch buffers, generate appropriate interrupts, and handle buffer overflow conditions.

- A computer system contains several I/O devices with different data transfer sizes and timing requirements. Design a pseudocode for an I/O scheduling algorithm that determines the order in which pending I/O requests should be serviced. The algorithm should consider device priority, request size, waiting time, latency requirements, and available bandwidth to improve overall I/O performance while preventing starvation.

- A high-performance embedded system uses multiple DMA-capable devices that may request memory access at the same time. Develop a pseudocode for a DMA channel arbitration mechanism that allocates memory bus access among competing DMA requests. The algorithm should consider request priority, transfer size, urgency, and fairness, while ensuring that high-priority transfers are completed within their required time constraints.

- A real-time system must communicate simultaneously with several devices that have different communication speeds and reliability requirements. Write a pseudocode for an adaptive I/O management mechanism that monitors device performance and dynamically changes the communication method or transfer parameters when congestion, delays, or errors are detected. The algorithm should monitor transfer status, detect performance degradation, adjust resource allocation, and maintain reliable communication with minimal CPU overhead.

Code of Conduct

I 	(Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment. I understand that any violation of academic integrity rules will result in disciplinary action.

## Signature of the Student:

### RUBRICS FOR CALCULATION

| Criteria | Weightage(%) | Level 4 – Exemplary (5 Marks) | Level 3 – Proficient (4 Marks) | Level 2 – Developing (2–3 Marks) | Level 1 – Beginning (0–1 Mark) |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Identifies all inputs, outputs, constraints accurately | Minor missing elements | Partial understanding | Incorrect interpretation |
| Algorithm Design | 30% | Complete, correct, and logically sound solution | Minor logical gaps | Partially correct logic | Incorrect approach |
| Use of Control Structures | 20% | Efficient and appropriate use of loops, conditions, functions/modules | Minor inefficiencies | Limited or basic usage | Incorrect or missing constructs |
| Pseudocode Structure | 10% | Well-structured, clear, consistent format, easy to follow | Mostly clear with minor issues | Difficult to follow in parts | Poorly structured and unclear |
| Completeness | 20% | Handles all cases; optimized approach | Handles most cases; reasonable efficiency | Handles basic cases only | Incomplete or inefficient |

## Marks Evaluation:

| Q. No | Problem Understanding (20) |  | Algorithm Design (30) |  | Use of Control Structures (20) | Pseudocode Structure (10) | Completeness (20) | Total (out of 100) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 |  |
| 2 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 |  |
| 3 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 |  |
| 4 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 |  |
| 5 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 |  |
|  | / 20 |  | /30 |  | / 20 | / 10 | / 20 | / 100 |

Course Coordinator	Faculty Incharge

Answers: Interrupt, DMA, and I/O Scheduling Pseudocode

Q1. Interrupt Scheduling for Multi-Sensor Real-Time Monitoring

The algorithm assigns each sensor a priority based on urgency and sampling rate, then services interrupts in priority order. A short interrupt service routine (ISR) just queues the work; the heavy processing is done afterward so the CPU is not blocked for long periods.

BEGIN

FOR each sensor S in SensorList:

Compute Priority(S) = W1*Urgency(S) + W2*(1/SamplingPeriod(S)) + W3*ProcessingLoad(S) END FOR

Sort SensorList by Priority (highest first) ON Interrupt Request from Sensor S:

IF CPU is busy servicing a HIGHER priority interrupt: Queue S's request in PendingQueue

ELSE:

Save current context (registers, program counter) Disable lower priority interrupts temporarily

Call ISR_for(S)	// short routine: read data, timestamp, ack device Enqueue(S.data) into ProcessingQueue

Enable interrupts Restore context

BACKGROUND TASK (runs continuously): WHILE PendingQueue is not empty:

Pick sensor with highest priority from PendingQueue Process its queued data (filtering, storage, alerts) Remove it from PendingQueue

IF PendingQueue length exceeds Threshold: Raise "CPU Overload Warning"

Temporarily drop or batch lowest-priority, non-critical samples

END

Q2. Double-Buffered DMA Mechanism

Two buffers, Buffer A and Buffer B, are used alternately. While the DMA controller fills one buffer from the I/O device, the CPU processes the other buffer that was already filled. This overlap prevents data loss when the incoming rate is faster than CPU processing.

BEGIN

Initialize BufferA, BufferB (empty) ActiveBuffer = BufferA

ReadyBuffer = NULL

START DMA transfer: Device -> ActiveBuffer ON DMA_Buffer_Full_Interrupt:

ReadyBuffer = ActiveBuffer IF ActiveBuffer == BufferA:

ActiveBuffer = BufferB ELSE:

ActiveBuffer = BufferA

IF ReadyBuffer is still being read by CPU (not yet consumed):

Raise "Buffer Overflow" interrupt

Discard oldest unread data OR pause incoming stream (flow control) ELSE:

START DMA transfer: Device -> ActiveBuffer	// switch immediately Notify CPU: "ReadyBuffer available for processing"

CPU PROCESS (parallel task): WHILE TRUE:

WAIT for "ReadyBuffer available" signal Process data in ReadyBuffer

Mark ReadyBuffer as consumed/empty

ON Transfer Complete (all data sent): Stop DMA

Flush remaining buffer to CPU

END

Q3. I/O Scheduling Algorithm for Multiple Devices

Each pending request is given a priority score combining device priority, waiting time, latency requirement, and request size. The request with the highest score is serviced next. Aging (adding weight for waiting time) prevents starvation of low-priority devices.

BEGIN

FOR each pending request R in RequestQueue: Score(R) = W1*DevicePriority(R)

+ W2*(1/LatencyRequirement(R))

+ W3*WaitingTime(R)	// aging factor, prevents starvation

- W4*RequestSize(R)	// larger transfers cost more time

+ W5*AvailableBandwidth(Device(R))

WHILE RequestQueue is not empty:

Sort RequestQueue by Score (highest first) R = RequestQueue.pop_highest()

IF Bandwidth(Device(R)) is sufficient AND Latency deadline not missed: Service R (perform transfer)

Remove R from RequestQueue ELSE:

Requeue R, increase its WaitingTime for next round

FOR each remaining request R' in RequestQueue: WaitingTime(R') += TimeElapsed

IF WaitingTime(R') > MaxWaitThreshold:

Boost Score(R') significantly	// anti-starvation aging

END

Q4. DMA Channel Arbitration Mechanism

When multiple DMA-capable devices request the bus at once, an arbiter grants access based on a weighted score of priority, urgency, and transfer size, while tracking how long each device has waited so that no device is indefinitely denied access (fairness).

BEGIN

FOR each DMA request D in RequestList: Score(D) = W1*Priority(D) + W2*Urgency(D)

+ W3*(1/TransferSize(D)) + W4*WaitTime(D) WHILE Bus is free AND RequestList is not empty:

Sort RequestList by Score (highest first) D = RequestList.pop_highest()

IF D.deadline is close AND Bus can be granted immediately: Grant bus to D

Perform DMA transfer for D

Release bus when D's transfer completes ELSE:

Requeue D

FOR each remaining request D' in RequestList: WaitTime(D') += TimeElapsed

IF WaitTime(D') > FairnessThreshold:

Boost Score(D') to guarantee eventual access

IF D missed its required time constraint: Raise "Deadline Violation" alert

Escalate D's priority for next arbitration cycle

END

Q5. Adaptive I/O Management for Multiple Devices

The system continuously monitors each communication channel's performance (throughput, error rate, delay). If it detects congestion, errors, or degradation, it dynamically adjusts the communication method or transfer parameters (rate, packet size, protocol) to maintain reliability with minimal CPU overhead.

BEGIN

FOR each connected Device Dv:

Initialize CommMethod(Dv) = DefaultMethod Initialize TransferParams(Dv) = DefaultParams

MONITOR LOOP (runs periodically, low overhead): FOR each Device Dv:

Measure Throughput(Dv), ErrorRate(Dv), Delay(Dv)

IF ErrorRate(Dv) > ErrorThreshold OR Delay(Dv) > DelayThreshold:

// performance degradation or congestion detected Reduce TransferRate(Dv)

Reduce PacketSize(Dv)

IF degradation persists over N cycles:

Switch CommMethod(Dv) to a more reliable/alternate mode

(e.g., polling -> interrupt-driven, or lower-speed reliable link)

ELSE IF Throughput(Dv) is stable AND ErrorRate(Dv) is low: Gradually increase TransferRate(Dv) toward optimal level

// recover performance when conditions improve Log status of Dv for diagnostics

Sleep(MonitorInterval)	// avoid busy-waiting, keep CPU overhead low

ON Critical Communication Failure with Dv: Attempt automatic reconnect / fallback method IF reconnect fails repeatedly:

Flag Dv as "Offline" and alert system

END